### Wiring Instructions

To set up this project, follow these wiring instructions:

#### Components:

1. **MPU6050 (Accelerometer and Gyroscope)**

2. **HX711 (Load Cell Amplifier)**

3. **GPS Module**

4. **ESP8266 (WiFi Module)**

#### Connections:

1. **MPU6050:**

- VCC to 3.3V or 5V (depending on your module's requirements)

- GND to GND

- SDA to D2 (or any other available GPIO pin)

- SCL to D1 (or any other available GPIO pin)

2. **HX711:**

- VCC to 3.3V

- GND to GND

- DT (Data) to D6

- SCK (Clock) to D7

3. **GPS Module:**

- VCC to 3.3V or 5V (depending on your module's requirements)

- GND to GND

- RX to D1 (TX of ESP8266)

- TX to D2 (RX of ESP8266)

4. **ESP8266:**

- VCC to 3.3V

- GND to GND

- GPIO D1 (connected to SCL of MPU6050)

- GPIO D2 (connected to SDA of MPU6050)

- GPIO D6 (connected to DT of HX711)

- GPIO D7 (connected to SCK of HX711)

### Project Explanation

#### Objective:

The project involves collecting data from various sensors (MPU6050 for steps and distance, HX711 for weight, and GPS for location) and sending this data to a Firebase Realtime Database using an ESP8266 module for remote monitoring.

#### Components:

1. **MPU6050:**

- Measures acceleration and gyroscope data.

- Used for detecting steps and calculating the distance covered.

2. **HX711:**

- Amplifies the signal from a load cell.

- Measures weight.

3. **GPS Module:**

- Provides location data including latitude, longitude, altitude, and speed.

- Calculates total distance traveled.

4. **ESP8266:**

- Connects to WiFi and communicates with Firebase.

- Sends collected sensor data to the Firebase Realtime Database.

### Code Explanation

1. **Setup Function:**

- Initializes Serial communication.

- Connects to WiFi.

- Initializes MPU6050, HX711, and GPS modules.

- Sets up power management (placeholder).

2. **Loop Function:**

- Uses a state machine to manage different states:

- **READ_SENSORS:** Reads data from MPU6050, HX711, and GPS.

- **PROCESS_DATA:** Placeholder for any additional data processing.

- **SEND_DATA:** Sends the collected data to Firebase.

- **ERROR_HANDLING:** Manages any errors that occur during operation.

3. **Functions:**

- **connectToWiFi:**

- Connects to the specified WiFi network and handles connection retries.

- **initializeSensors:**

- Initializes the MPU6050, HX711, and GPS modules.

- Loads calibration data for HX711 from EEPROM.

- **readMPU6050Data:**

- Reads acceleration data.

- Detects steps and calculates the distance covered.

- **readHX711Data:**

- Reads weight data.

- Sends weight data to Firebase.

- **readGPSData:**

- Reads and processes GPS data.

- Sends location data to Firebase.

- **sendFirebaseData:**

- Sends data to Firebase at the specified path.

- **displayGPSInfo:**

- Displays and sends GPS information (latitude, longitude, altitude, speed).

- **handleError:**

- Handles and displays error messages.

### Detailed Code

```cpp

#include <Wire.h>

#include <MPU6050.h>

#include <HX711_ADC.h>

#include <EEPROM.h>

#include <TinyGPS++.h>

#include <SoftwareSerial.h>

#include <ESP8266WiFi.h>

#include <FirebaseESP8266.h>

// Constants and configurations

const int stepThreshold = 11000;

const float stepsPerMeter = 1.32;

const char* ssid = "your-SSID";

const char* password = "your-PASSWORD";

#define FIREBASE_HOST "https://gradus-77593-default-rtdb.europe-west1.firebasedatabase.app/"

#define FIREBASE_AUTH "your-database-secret"

// Pins

const int HX711_dout = D6;

const int HX711_sck = D7;

const int RXPin = D2, TXPin = D1;

const uint32_t GPSBaud = 9600;

const int calVal_eepromAddress = 0;

// Global variables

MPU6050 mpu;

HX711_ADC LoadCell(HX711_dout, HX711_sck);

TinyGPSPlus gps;

SoftwareSerial ss(RXPin, TXPin);

FirebaseData firebaseData;

int16_t ax, ay, az;

int stepCount = 0;

float distanceCovered = 0.0;

unsigned long t = 0;

double prevLat = 0.0, prevLng = 0.0, totalDistance = 0.0;

// Function declarations

void connectToWiFi();

void initializeSensors();

void readMPU6050Data();

void readHX711Data();

void readGPSData();

void sendFirebaseData(const String& path, float value);

void displayGPSInfo();

void handleError(const String& errorMsg);

// Power management setup

void powerManagementSetup() {

// Placeholder for power management setup if needed (e.g., sleep modes)

}

// State machine setup

enum State {

READ_SENSORS,

PROCESS_DATA,

SEND_DATA,

ERROR_HANDLING

};

State currentState = READ_SENSORS;

void setup() {

Serial.begin(115200);

connectToWiFi();

Firebase.begin(FIREBASE_HOST, FIREBASE_AUTH);

Firebase.reconnectWiFi(true);

initializeSensors();

powerManagementSetup();

}

void loop() {

switch (currentState) {

case READ_SENSORS:

readMPU6050Data();

readHX711Data();

readGPSData();

currentState = PROCESS_DATA;

break;

case PROCESS_DATA:

// Process sensor data (placeholder if additional processing is needed)

currentState = SEND_DATA;

break;

case SEND_DATA:

sendFirebaseData("/steps", stepCount);

sendFirebaseData("/distance", distanceCovered);

currentState = READ_SENSORS;

break;

case ERROR_HANDLING:

// Handle any errors (placeholder)

currentState = READ_SENSORS;

break;

}

}

void connectToWiFi() {

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi...");

int retryCount = 0;

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

retryCount++;

if (retryCount > 60) {

handleError("Failed to connect to WiFi.");

return;

}

}

Serial.println(" connected.");

}

void initializeSensors() {

Wire.begin();

mpu.initialize();

if (!mpu.testConnection()) {

handleError("MPU6050 connection failed");

return;

}

Serial.println("MPU6050 initialized successfully");

float calibrationValue = 696.0;

EEPROM.begin(512);

EEPROM.get(calVal_eepromAddress, calibrationValue);

LoadCell.begin();

unsigned long stabilizingTime = 2000;

bool performTare = true;

LoadCell.start(stabilizingTime, performTare);

if (LoadCell.getTareTimeoutFlag()) {

handleError("HX711 initialization timeout, check wiring.");

return;

} else {

LoadCell.setCalFactor(calibrationValue);

Serial.println("HX711 initialization complete");

}

ss.begin(GPSBaud);

Serial.println("GPS module initialized");

}

void readMPU6050Data() {

mpu.getAcceleration(&ax, &ay, &az);

long magnitudeSquared = (long)ax * ax + (long)ay * ay + (long)az * az;

if (magnitudeSquared > (long)stepThreshold * stepThreshold) {

stepCount++;

distanceCovered = stepCount / stepsPerMeter;

}

Serial.print("Steps: ");

Serial.println(stepCount);

Serial.print("Distance: ");

Serial.println(distanceCovered);

}

void readHX711Data() {

static bool newDataReady = false;

const int serialPrintInterval = 500;

if (LoadCell.update()) newDataReady = true;

if (newDataReady) {

if (millis() > t + serialPrintInterval) {

float weight = LoadCell.getData();

Serial.print("Weight: ");

Serial.println(weight);

newDataReady = false;

t = millis();

sendFirebaseData("/weight", weight);

}

}

}

void readGPSData() {

while (ss.available() > 0) {

if (gps.encode(ss.read())) {

displayGPSInfo();

}

}

if (millis() > 5000 && gps.charsProcessed() < 10) {

handleError("No GPS detected: check wiring.");

}

}

void sendFirebaseData(const String& path, float value) {

if (Firebase.pushFloat(firebaseData, path, value)) {

Serial.print(path);

Serial.println("

data sent to Firebase");

} else {

Serial.print("Firebase push failed: ");

Serial.println(firebaseData.errorReason());

currentState = ERROR_HANDLING;

}

}

void displayGPSInfo() {

if (gps.location.isValid()) {

double currentLat = gps.location.lat();

double currentLng = gps.location.lng();

Serial.print("Location: ");

Serial.print(currentLat, 6);

Serial.print(",");

Serial.println(currentLng, 6);

if (prevLat != 0.0 && prevLng != 0.0) {

double distance = TinyGPSPlus::distanceBetween(prevLat, prevLng, currentLat, currentLng);

totalDistance += distance;

}

prevLat = currentLat;

prevLng = currentLng;

sendFirebaseData("/gps/latitude", currentLat);

sendFirebaseData("/gps/longitude", currentLng);

sendFirebaseData("/gps/totalDistance", totalDistance);

} else {

Serial.println("INVALID Location");

}

Serial.print(" Date: ");

if (gps.date.isValid()) {

Serial.print(gps.date.month());

Serial.print("/");

Serial.print(gps.date.day());

Serial.print("/");

Serial.print(gps.date.year());

} else {

Serial.print("INVALID Date");

}

Serial.print(" Time: ");

if (gps.time.isValid()) {

if (gps.time.hour() < 10) Serial.print("0");

Serial.print(gps.time.hour());

Serial.print(":");

if (gps.time.minute() < 10) Serial.print("0");

Serial.print(gps.time.minute());

Serial.print(":");

if (gps.time.second() < 10) Serial.print("0");

Serial.print(gps.time.second());

} else {

Serial.print("INVALID Time");

}

Serial.print(" Altitude: ");

if (gps.altitude.isValid()) {

Serial.print(gps.altitude.meters());

sendFirebaseData("/gps/altitude", gps.altitude.meters());

} else {

Serial.print("INVALID Altitude");

}

Serial.print(" Speed: ");

if (gps.speed.isValid()) {

Serial.print(gps.speed.mps());

sendFirebaseData("/gps/speed", gps.speed.mps());

} else {

Serial.print("INVALID Speed");

}

Serial.print(" Distance: ");

Serial.println(totalDistance, 6);

}

void handleError(const String& errorMsg) {

Serial.println(errorMsg);

// Add further error handling and recovery actions if needed

delay(5000);  // Delay to allow user to see the error message

}

```

### Report Summary

This project demonstrates a comprehensive IoT application using multiple sensors to monitor physical activity and environment data. The system includes an MPU6050 for step counting, an HX711 for weight measurement, and a GPS module for location tracking. Data from these sensors is collected and transmitted to a Firebase Realtime Database using an ESP8266 WiFi module, allowing remote monitoring and analysis.

The provided code initializes and manages these sensors, collects their data, and sends it to Firebase. It incorporates a state machine for scalable and maintainable code structure, with distinct states for reading sensors, processing data, sending data, and handling errors.

This project can be expanded by adding more sensors or integrating additional features such as power management techniques, further data processing, or advanced error recovery mechanisms.